Volume 3.2_Terms of Reference

Дорожній контролер з GPRS модулем та їз системою точного часу GPS

(на 24 силових виходів)

Технічні та якісні вимоги до дорожніх контролерів

| 1 | Дорожні контролери (ДК) повинні бути виготовлені у відповідності до вимог ДСТУ 4157 – 2003. Повинен бути представлений сертифікат відповідності технічних характеристик дорожніх контролерів вимогам ДСТУ 4157-2003 (п.п. 5.2; 6.1.1 - 6.1.3; 6.1..12 - 6.1.27; 6.2 - 6.4; 7). |
| --- | --- |
| 2 | Живлення ДК - змінний струм ~ 220В (-15%,+10%), 50 Гц ±2% |
| 3 | Споживча потужність ДК (без ламп освітлення) - до 20 Вт |
| 4 | Кількість силових виходів – згідно з специфікацією закупівлі від 16 до 40 |
| 5 | Максимальне навантаження на силовий канал - не менше 2 А |
| 6 | Мінімальне значення активного навантаження на один силовий канал, яке реєструється – 20 мА при АС 220 В (~4,5Вт ) |
| 7 | Для вимірювання навантаження в кожному силовому каналі застосувати лише один фазний провід. Нульовий провід застосувати один, загальний для всіх каналів |
| 8 | Всі канали повинні бути контрольованими |
| 9 | ДК та всі канали повинні бути захищені пристроями грозозахисту |
| 10 | Кількість фаз руху - не менше 8 |
| 11 | Похибка відліку інтервалів часу - не більше 2 % |
| 12 | Кількість часових програм - не менше 8 |
| 13 | Кількість підключень ТВП (табло виклику пішоходів) - не менше 2. Напруга на кнопці виклику не більше 12 В постійного струму |
| 14 | Повинна бути забезпечена можливість формування сигналу «ЧЕКАЙТЕ» на одному з силових виходів при отриманні сигналу з ТВП |
| 15 | Забезпечення режиму “ЗЕЛЕНА ВУЛИЦЯ”- за командою обслуговуючого персоналу при умові підключення до Центрального або мобільного пульта управління |
| 16 | Протокол обміну інформацією ДК з Центральним пультом управління (ЦПУ) повинен бути відкритим*. Надається Замовнику з правом передачі стороннім підприємствам для виконання проектних та пусконалагоджувальних робіт з впровадження АСКДР |
| 17 | З ДК повинна бути поставлена програмна система підготовки даних світлофорних об’єктів (СО). Мінімальний набір функцій системи: а)формування геометрії СО; б)формування режиму роботи СО; в)програмування ДК; г)роздруківка програми у встановленому форматі; д)ведення бази даних СО та архіву. |
| 18 | Група захисту ДК (від проникнення. води, пилу) - не нижче ІР– 54 згідно ДСТУ 14254-80 |
| 19 | Діапазон робочих температур – від - 40 + 60 ºС |
| 20 | ДК повинен бути захищеним від несанкціонованого проникнення сигналізацією на Центральний пункт управління (ЦПУ). |
| 21 | В ДК повинна бути забезпечена можливість застосування наступних засобів зв’язку з ЦПУ: а) дротовий; б) GPRS; в) Ethernet |
| 22 | ДК повинен бути оснащений комплектом GPRS-зв’язку |
| 23 | ДК повинен бути оснащений пристроєм синхронізації часу за сигналами GPS |
| 24 | Захист від уражень електричним струмом. |
| 25 | В ДК повинна бути забезпечена можливість безпосереднього керування дорожнім контролером індикацією табло зворотного відліку світлофорів на світлофорному об’єкті з застосуванням інтерфейсу RS-485 і протоколу зв’язку, наведеного в Примітці** |
| 26 | ДК повинен забезпечувати контроль перегорання червоних сигналів світлофорів з автоматичним переведенням світлофорної сигналізації у режим миготіння жовтих сигналів (ЖМ), якщо перегоріли всі червоні сигнали відокремленого напрямку (основні та дублюючі) |
| 27 | ДК повинен забезпечувати блокування одночасного вмикання комбінацій сигналів світлофорів, які не передбачені запрограмованою схемою організації дорожнього руху - захист при виникненні конфліктної ситуації з переходом в режим відключення сигналів світлофорів (ОС) |
| 28 | Повинна бути забезпечена фіксація можливих несправностей ДК зі збереженням результатів в енергонезалежній пам’яті. Доступ до інформації - за допомогою ноутбуку, зняття інформації на флеш – пам'ять, або по каналу Bluetooth на мобільний телефон |
| 29 | Час стартового запуску ДК не повинен перевищувати 20 секунд після подачі електроживлення у всьому діапазоні робочих температур |
| 30 | Корпус контролера повинен мати антивандальне виготовлення з нержавіючої сталі з лакофарбовим покриттям, яке забезпечить можливість експлуатації ДК в умовах міських мереж не менше 10 років. |
| 31 | Середнє напрацювання на відмову не менше 12 000 годин. |
| 32 | Термін експлуатації контролера не менше 10 років. |
| 33 | Гарантійний термін - не менше 3 років від дати вводу в експлуатацію. |

Дорожні контролери планується підключити до діючої в м. Харків автоматизованої системи керування дорожнім рухом, тому запропонований контролер повинен підтримувати діючий протокол обміну інформацією.

Дана вимоги обумовлена тим, що діюча в м. Харків автоматизована система керування дорожнім рухом підтримується діючим відкритим протоколом. Закупівля контролерів, що не підтримують діючий протокол обміну інформацією, може призвести до перепрограмування всієї системи, що в свою чергу призведе до питомих та необґрунтованих витрат коштів підприємства.

Учасник повинен використовувати відкритий протокол обміну інформацією ДК з Центральним пультом управління по GPRS, який підтримує діючий протокол обміну інформацією.

Діючий відкритий протокол обміну інформацією ДК з Центральним пультом управління по GPRS

### Основні позначення і скорочення

ТИП УПРАВЛІННЯ:

КУ - координоване управління

ДУ - диспетчерське управління

ЛУ - локальне управління

РУ - ручне управління

РЕЖИМ РОБОТИ :

ВС - відключення світлофора

ЖМ - жовте мигання

КЧ - кругом червоний

ДП - добова програма

РДП - резервна добова програма

СФ - спеціальна фаза

ФДП – фіксована добова програма, відрізняється від ДП(координованої) тим, що порядок та довжина фаз беруться з локальної програми з тим самим номером.

### Опис протоколу

#### Формат пакетів

Для зв'язку по діючому протоколу  використовуються: радіоканал, Ethernet, GPRS.

Швидкість обміну – 1200 бод, ємкість каналу – 8біт, біт непарності та 
1 стоповий біт;

Загальний формат:

| маркер | адреса 1 | адреса 2 | код команди | довжина | дані | кс |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Поле 1: довжина 1 байт, код - 0x3A, початок тексту.

Поле 2: довжина 1 байт, мережева адреса абонента - приймача (FF - прийняти усім).

Поле 3: довжина 1 байт, мережева адреса абонента - джерела.

Поле 4: довжина 1 байт, код команди.

D7 у байті коду команди = 0 - запит; 1 - відповідь.

Поле 5: довжина 1 байт, код довжини.

Поле 6: довжина від 0 до 255 байт, дані.

Поле 7: довжина 1 байт, контрольна сума (XOR полів 2...6).

#### Сервісні команди

##### Список сервісних команд

| код команди | опис команди |
| --- | --- |
| 0x03 | РЕСТАРТ |
| 0x51 | ТЕСТ ЗВ'ЯЗКУ |
| 0x52 | ВСТАНОВИТИ ЧАС |
| 0x53 | ПРОЧИТАТИ ЧАС |

##### Описи сервісних команд

| команда | 0x03 - РЕСТАРТ |
| --- | --- |
| відповідь | 1 байт: старша тетрада: 0 - команда прийнята, але не виконана 1 - команда виконана 2 - команда не підтримується цим типом контролера 3 - відхилена (виконується команда більш високого пріоритету) молодша тетрада: резерв |

| команда | 0x51 - ТЕСТ ЗВ'ЯЗКУ |
| --- | --- |
| довжина | 10 байт |
| дані | довільні |
| відповідь | повторює запит |

| команда | 0x52 - ВСТАНОВИТИ ЧАС |
| --- | --- |
| довжина | 7 байт |
| дані | секунди, хвилини, години, день тижня, число, місяць, рік. (у двійково-десятковому виді) |
| відповідь | довжина - 0 байт |

| команда | 0x53 - ПРОЧИТАТИ ЧАС |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 7 байт дані - секунди, хвилини, години, день тижня, число, місяць, рік. (у двійково-десятковому виді) |

#### Команди опитування стану

##### Список команд опитування стану

| код команди | опис команди |
| --- | --- |
| 0x60 | ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ |
| 0x70 | ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ (З ЧАСОВОЮ МІТКОЮ) |
| 0x42 | СТАН ВИВОДІВ (КАНАЛІВ) |
| 0x66 | КОНФЛІКТИ В НАПРЯМКАХ |
| 0x67 | ПЕРЕГОРАННЯ КАНАЛУ |
| 0x68 | КІЛЬКІСТЬ ЛАМП, ЩО ПЕРЕГОРІЛИ |

##### Описи сервісних команд

| команда | 0x60 - ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 5 байт дані - 1 байт: D7...D6 - тип управління 0b00 - КУ 0b01 - ДУ 0b10 - ЛУ 0b11 - РУ D5.D3 - режим роботи 0b000 - ВС 0b001 - ЖМ 0b010 - КЧ 0b011 - ДП 0b100 - РДП 0b101 - СФ 0b110, 0b111 - резерв 0b110 - ФДП D2 - коригування циклу 1 - початок циклу контролера не співпадає з заданою тимчасовою міткою. Контролер працюватиме по мінімальним годинам фаз до усунення відмінності. 0 - корекція не проводиться. D1 - такт фази 0 - основний такт 1 - проміжний такт D0 - старший біт номера ДП 2 байт: D7…D4 - номер поточної ДП 0x00.0x1F - номер ДП 1...32 D3.D0 - номер поточної фази 0x0.0xF - номер фази 1...16 3 байт: тривалість поточного основного/проміжного такту в с. 4 байт: D7 - відкриті двері D6 - конфлікт D5 - обрив червоного D4 - наявність ламп, що перегоріли D3 - немає мережі ~220В D2 - внутрішня несправність контролера D1 - залипання ТВП2 (1-кнопка залипнула, 0-віджата, за наявності фази ТВП) D0 - залипання ТВП1 (1-кнопка залипнула, 0-віджата, за наявності фази ТВП) 5 байт: D0 - ознака стартового промтакту (Перехід з КЧ у фазу, якщо “1”, то йде промтакт, при чому раніше не було фази. Такий стан може бути при переході в циклічну програму після ВС або ЖМ. D4 – помилка ЛЗ (лінії зв’язку) контролера Інші біти - резерв |

| команда | 0x70 - ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ (З ЧАСОВОЮ МІТКОЮ) |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 8 байт дані - 1…3 байти - знімок часу на момент формування 1 байт: с. в DCB 2 байт: хв. в DCB 3 байт: година в DCB 4…9 байти - інформація про стан. Формат такий самий, як відповідь на команду 0x60. |

| команда | 0x42 - СТАН ВИВОДІВ (КАНАЛІВ) |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 6(8) байт (довжина даних залежить від моделі ДК) дані - 1…6(8) байти - бітове поле станів усіх каналів 1 байт: канали 1...8 D7 - 8-й канал … D0 - 1-й канал 2 байт: канали 9…16 D7 - 16-й канал … D0 - 9-й канал |

| команда | 0x66 - КОНФЛІКТИ В НАПРЯМАХ |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 2 байти дані - 1..2 байти - номери конфліктних виходів (2 номери), при не виявленні конфлікту 1-й і 2-ою байти = 0 |

| команда | 0x67 - ПЕРЕГОРАННЯ КАНАЛУ |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 8 байт дані - 1..8 байт - перегорання каналу. "Бітове поле" (64 канали) 0 - канал цілий 1 - канал перегорів |

| команда | 0x68 - КІЛЬКІСТЬ ЛАМП, ЩО ПЕРЕГОРІЛИ |
| --- | --- |
| довжина | 0 байт |
| відповідь | довжина - 0 до 32 байт дані - по тетрадам: 0 - усі лампи в каналі цілі 1..14 - кількість ламп, що перегоріли 0xF - перегоріли усі лампи Номер каналу визначається номером тетради. |

#### Команди керування

##### Список команд керування

| код команди | опис команди |
| --- | --- |
| 0x61 | ПЕРЕХІД В РЕЖИМ ПО ЧАСУ |
| 0x62 | ПЕРЕХІД В РЕЖИМ НЕГАЙНО |
| 0x63 | УСТАНОВКА В ФАЗУ |
| 0x64 | ПЕРЕЙТИ В ЛУ |
| 0x6D | УПРАВЛІННЯ ПЕРЕФЕРІЄЮ |

Описи команд управління

| команда | 0x61 - ПЕРЕХІД В РЕЖИМ ПО ЧАСУ |
| --- | --- |
| довжина | 4…28 байт |
| дані | 1..3 байти - час включення в режим 1 байт: с. в DCB 2 байт: хв. в DCB 3 байт: година в DCB 0xFFFFFF - виконати по закінченню Тmin 4 байт: D7 - тип управління 0 - КУ 1 - ДУ D6...D5 - режим роботи 0b00 - ВC 0b01 - ЖМ 0b10 - КЧ 0b11 – ДП (ФДП) D4...D0 - номер ДП (ФДП) 0x00.0x1F - 1...32 ДП При відсутності зазначених нижче байтів [5-тий і далі) з номерами фаз по тетрадах і тривалості Тосн. фаз побайтно, режим ДП вважається режимом ФДП - це як і ДП, але з послідовністю фаз і тривалістю, узятими зі списку фіксованих добових програм контролера з відповідним номером, і тільки у тому випадку, якщо вказана фіксована програма є циклічною. Інакше команда не виконується. В іншому випадку нижче зазначені байти вказують на порядок чергування фаз та їх тривалість. 5...(4+N) байти: номери фаз по тетрадах, у порядку чергування в циклі 0x0...0xF - 1…16 фаз (5+N)…(4+K) байти: тривалість Тосн. фаз побайтно N = кількість фаз / 2 K = N + кількість фаз |
| відповідь | довжина - 1...9 байт дані - 1 байт: старша тетрада: 0 - команда прийнята (у молодшій тетраді к-ть фаз) 1 - команда виконана (у молодшій тетраді к-ть фаз) 2 - команда не підтримується цим типом контролера 3 - відхилена (виконується команда більш високого пріоритету) 4 - відхилена (к-ть фаз більше ніж прописано в контролері) 5 - відхилена (Tmin > Tфазы, у фазі X) молодша тетрада: (відсутня в режимі ФДП) Трактується залежно від значення в старшій тетраді: старша; 0..1; 2..4; 5. молодша; к-ть фаз 0x0...0xF - 1...16 фаз; 0b0000; номер фази Х 0x0.0xF - 1...16 фаза. 2...(N+1) байти: (відсутні в режимі ФДП) Номери фаз по тетрадах, у порядку дотримання в циклі (є присутнім, тільки якщо в старшій тетраді 1 байта - 0...1) 0x0...0xF - 1...16 фаза N = к-ть фаз / 2 |

| команда | 0x62 - ПЕРЕХІД В РЕЖИМ НЕГАЙНО |
| --- | --- |
| довжина | 1...25 байт |
| дані | формат даних аналогічний формату в команді 0x61, за винятком відсутності 3 байт, задаючих час включення. |

| команда | 0x63 - УСТАНОВКА У ФАЗУ |
| --- | --- |
| довжина | 4 байти |
| дані | 1...3 байти - час включення в режим 1 байт: с. в DCB 2 байт: хв. в DCB 3 байт: година в DCB 0xFFFFFF - приступити до переходу негайно 4 байт: D7...D4 - резерв D3...D0 - номер фази (0x0...0xF - 1...16 фаза) |
| відповідь | довжина - 1 байт дані - 1 байт: старша тетрада: 0 - команда прийнята 1 - команда виконана 2 - команда не підтримується цим типом контролера 3 - відхилена (виконується команда більш високого пріоритету) молодша тетрада: резерв |

| команда | 0x64 - ПЕРЕЙТИ В ЛУ |
| --- | --- |
| довжина | 0 байта |
| відповідь | довжина - 1 байт дані - 1 байт: старша тетрада: 0 - команда прийнята 1 - команда виконана 2 - команда не підтримується цим типом контролера 3 - відхилена (виконується команда більш високого пріоритету) молодша тетрада: резерв |

| команда | 0x6D - Управління периферією |
| --- | --- |
| довжина | змінна (альтернативна команда) Формат поля даних альтернативної команди 1; 2; 2; 2; 2; 2. 1. підкоманда (при відповіді не модифікується) 2. дані (у разі запиту - параметри) За умовчанням (якщо в описі не вказано явно) для багатобайтних значень відповіді вважається, що перший прийнятий байт (тобто байт, що має менший індекс у буфері відповіді) містить молодший байт значення і так далі. Усередині байту молодший біт відповідає молодшій адресі. Усі коди підкоманд вказані в шістнадцятиричному виді. |
|  | Підкоманда 01 - управління знаками рекомендованої швидкості |
| запит | 2 байти 1 байт - швидкість для першого знаку (у HEX) 2 байт - швидкість для другого знаку (у HEX) Приклад запиту : 6D 03 01 32 3С - включити індикацію 50 км/год для першого знаку і 60 км/год для другого |
| відповідь | довжина - 1 байт 1 байт: старша тетрада: 0 - команда прийнята 1 - команда виконана 2 - команда не підтримується цим типом контролера молодша тетрада: резерв |

#### Альтернативні команди 34 (hex)

##### Формат поля даних альтернативної команди

| 1 | 2 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- |

1. підкоманда (при відповіді не модифікується)

2. дані (у разі запиту - параметри)

За умовчанням (якщо в описі не вказано явно) для багатобайтних значень відповіді вважається, що перший прийнятий байт (тобто байт, що має менший індекс у буфері відповіді) містить молодший байт значення і так далі. Усередині байту молодший біт відповідає молодшій адресі.

Усі коди підкоманд вказані в шістнадцятиричному виді.

##### Список підкоманд альтернативної команди

| код підкоманди | опис підкоманди |
| --- | --- |
| 0-29 | Резерв |
| 30 | Мережева адреса і протокол |
| 31 | Стан контролера і дзеркало виводів |
| 32-49 | Резерв |
| 50 | Розширене опитування стану |
| 52-59 | Резерв |
| 5A | Зчитування датчиків напруги |
| 5C | читання розрядів портів |
| 5D - FF | Резерв |

##### Описи підкоманд альтернативної команди

| команда | 30 – МЕРЕЖЕВА АДРЕСА І ПРОТОКОЛ |
| --- | --- |
| параметри | Відсутні |
|  | 1-й байт - мережевий протокол 2-й байт - мережева адреса (молодший байт) 3-й байт - мережева адреса (старший байт) |
| примітка | Мережева адреса може бути однобайтною |

| команда | 31 – СТАН КОНТРОЛЛЕРА І ДЗЕРКАЛО ВИВОДІВ |
| --- | --- |
| параметри | відсутні |
| відповідь | 1-й. 5-й байти - ідентичні п’ятибайтній відповіді команди «0 x60 - ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ» 6-й байт - номінальна кількість каналів контролера 7-й. 14-й байти - 8-байтна, побітна маска стану каналів. Встановлений біт означає включений канал. |

| команда | 50 – ОПИТУВАННЯ СТАНУ, РОЗШИРЕНЕ |
| --- | --- |
| параметри | Відсутні |
| відповідь | 1-й байт D7…D6 тип управління 00 - ЛУ 01 - ДУ 10 - КУ 11 - РУ D5 - Відсутність даних прив'язки (1 - немає даних) D4 - ВС (силовий ключ, 1 - вимкнений силовий ключ) D3 - Ознака проміжного такту (1 - проміжний такт) D2 – Признак несправності, яка призводить до зміни режиму роботи (Це такі несправності, як конфлікт зелених, обрив червоного, КЗ, завищена чи занижена напруга живлення та інші. 1 – є несправність, 0 – немає несправності) D1...D0 Режим роботи 00 - Режим регулювання (не ВС, не ЖМ, не КЧ) 01 - ВС (вимкнені канали, 1 - канали вимкнені) 10 - ЖМ 11 - Кругом червоний 2-й байт: D7 - тумблер SET1 D6 - тумблер SET2 D5 - тумблер SET3 D4 - тумблер SET4 D3 - тумблер ВС D2 - тумблер ЖМ D1 - двері D0 - ознака завищеної або заниженої напруги живлення (1 –напруга завищена або занижена; 0 - напруга в межах норми) 3-й байт: D7 - ТПВ1 логічне D6 - ТВП1 фізичне D5 - ТВП2 логічне D4 - ТВП2 фізичне D3.D0 - джерело поточного режиму управління 0000 - Немає «зовнішнього» управління (контролер в локальному управлінні) 0001 - Локальний центр 0010 - АСУ радіо 0011 - АСУ СТАРТ 0100 - Пульт 0101 - Тумблери 0110 - Зовнішні тумблери 0111 – Поліцейський пульт (пульт спец призначення) 4-й байт: D7 - резерв D6 - резерв D5 - резерв D4 - резерв D3 - резерв D2 - резерв D1 - резерв D0 - Ознака коригування циклу (1 - проводиться коригування циклу, 0 - не проводиться) 5 байт: номер виконуваної програми (0 - немає програми, 1-24 - номер програми) 6 байт: номер фази (0 - немає фази, 1-16 - номер фази) 7 байт: поточний час етапу програми (для Тосн відлік вгору, для промтакта відлік вниз) 8, 9 байти: напруга мережі 10, 11 байти: напруга батареї 12, 13, 14, 15, 16, 17, 18 байти: секунди, хвилини, години, день тижня, число, місяць, рік в двійково-десятковому виді 19 байт: номер обірваного червоного (0 - немає обриву) 20, 21 байти: номери конфліктуючих зелених каналів (якщо по нулях - конфліктів немає; якщо конфлікт в одному каналі - тоді в молодшому байті номер конфліктуючого каналу, старший байт дорівнює 0; якщо конфлікт в двох каналах - тоді в обох байтах номера конфліктуючих каналів; 1 - перший канал) 22 байт: номер каналу, в якому спостерігається КЗ (0 - немає КЗ, 1 - перший канал) 23 – 30 байти: бітова маска стану управління в кожному каналі (D0 - молодший канал, 0 - викл, 1 - вкл) 31-38 байти: бітова маска значення струму в кожному каналі (D0 - молодший канал, 0 - викл, 1 - вкл) 39-46 байти: бітова маска значення напруги в кожному каналі (D0 - молодший канал, 0 - викл, 1 - вкл) |

| команда | 5A - ЧИТАННЯ ДАТЧИКІВ НАПРУГИ |
| --- | --- |
| параметри | 1-й байт - вид запиту 0-напруга мережі і батареї 1-напруга мережі і батареї і 1-32-го каналів 2-напруга мережі і батареї і 33-64-го каналів 3-напруга 1-32-го каналів 4-напруга 33-64-го каналів |
| відповідь | Одноплатний: масив двобайтових значень згідно із запитом 1-я пара - напруга мережі 2-я пара - напруга батареї (напруга каналів йде в порядку зростання номерів каналів) SPI - КДК: масив двобайтових значень згідно із запитом 1-я пара - напруга мережі 2-я пара - напруга батареї TWI - КДК: пішохідний (не відповідають) |

| команда | 5C - ЧИТАННЯ РОЗРЯДІВ ПОРТІВ |
| --- | --- |
|  | відсутні |
| відповідь | 1-й байт: 1-й біт - тумблер SET1 2-й біт - тумблер SET2 3-й біт - тумблер SET3 4-й біт - тумблер SET4 5-й біт - тумблер ВС 6-й біт - Стан лінії START. STARTIN 7-й біт - Стан лінії START. DATAIN 8-й біт - ЖМ |
|  | 2-й байт: 1-й біт - стан сигналу RXDI 2-й біт - стан сигналу DET1 3-й біт - ТВП1 4-й біт - ТВП2 5-й біт – двері |

**Примітка. Опис протоколу обміну між дорожніми контролерами і табло зворотного відліку часу для світлофорів.

Керовані табло зворотного відліку часу призначені для відображення значень залишку тривалості фази при змінній тривалості фаз в циклі. Зміна тривалості фаз може відбуватися при адаптивному управлінні, зокрема при управлінні від кнопки виклику пішохідної (ТВП).

Управління здійснюється шляхом передачі значення секунд, що відображаються на табло відліку по інтерфейсу RS-485. Відповідь від табло звіту не передається.

У протоколі передбачено, що кожне табло відліку має власну адресу. Синтаксис протоколу дозволяє адресувати до 32 табло відліку.

Формат байту

Швидкість передачі - 115200 бод.

Метод передачі - асинхронний

Кількість біт - 8

Стоп біти - 2

Контроль - по непарних

Формат кадру

| № | Значення | Опис |
| --- | --- | --- |
| 0 | FF | Фіксовані значення |
| 1 | 3A |  |
| 2 | 00 |  |
| 3 | 3E |  |
| 4 | 83 |  |
| 5 | 22 |  |
| 6 | 34 |  |
| 7 | Digit 1 | Значення відліку для 1-ї адреси табло |
| … | … |  |
| ... | ... |  |
| 38 | Digit 32 | Значення відліку для 32-ї адреси табло |
| 39 | Brightness | Значення яскравості світіння світлофора |
| 40 | CRC | Контрольна сума |
| 41 | FF |  |

Дані передаються в HEX (шістнадцятеричному вигляді).

Digit (Значення відліку) знаходяться в інтервалі 1-99 сек (1 - 63 в HEX). У разі відсутності необхідності відліку для даного світлофора значення дорівнює 0.

Brightness (Значення яскравості світіння) знаходяться в інтервалі 0 - 255 (0 - FF в HEX). Може не використовуватися.

CRC (Контрольна сума). Обчислюється порозрядним складанням по модулю 2 (XOR) числа "AB" і значень полів 7-39.

Road Controller with a GPRS Module and a GPS-Based Precise Time System

(with 24 power outputs)

Technical and Quality Requirements for Traffic Controllers

| 1 | Traffic controllers (TC) must be manufactured in accordance with the requirements of DSTU 4157–2003. A certificate of conformity must be provided, confirming that the technical characteristics of the traffic controllers meet the requirements of DSTU 4157-2003 (sections 5.2; 6.1.1–6.1.3; 6.1.12–6.1.27; 6.2–6.4; 7). |
| --- | --- |
| 2 | TC Power Supply - AC ~220 V (-15%, +10%), 50 Hz ±2% |
| 3 | Power consumption of the TC (excluding lighting bulbs) — up to 20 W |
| 4 | Number of power outputs: 16 to 40, as specified in the procurement specifications |
| 5 | The maximum load on the power channel is at least 2 A |
| 6 | The minimum active load per power channel that is recorded is 20 mA at 220 V AC (~4.5 W) |
| 7 | Use only one phase wire to measure the load in each power channel. Use a single neutral wire shared by all channels. |
| 8 | All channels must be controlled |
| 9 | TC and all circuits must be protected by surge protection devices |
| 10 | Number of movement phases: at least 8 |
| 11 | Time interval measurement error—no more than 2% |
| 12 | Number of time-based programs: at least 8 |
| 13 | Number of PCP (pedestrian call panel) connections: at least 2. Voltage at the call button: no more than 12 V DC |
| 14 | It must be possible to generate a “WAIT” signal on one of the power outputs upon receiving a signal from the PCP |
| 15 | Enabling “GREEN STREET” mode—upon command from service personnel, provided the system is connected to the central or mobile control panel |
| 16 | The protocol for exchanging information between the TC and the Central Control Unit (CCU) must be open*. It is provided to the Customer with the right to transfer it to third-party companies for the performance of design and commissioning work related to the implementation of the ASKDR system. |
| 17 | TC must be supplied with a software system for preparing traffic signal object (TSO) data. The system’s minimum set of functions includes: a) generating the geometry of the TSO; b) configuring the TSO’s operating mode; c) programming the CU; d) printing the program in the specified format; e) maintaining the TSO database and archive. |
| 18 | TC protection class (against water and dust ingress) — not lower than IP–54 in accordance with DSTU 14254-80 |
| 19 | Operating temperature range: -40 to +60 °C |
| 20 | TC must be protected against unauthorized access by an alarm system connected to the Central Control Station (CCS). |
| 21 | TC must support the following methods of communication with the CCS: a) wired; b) GPRS; c) Ethernet |
| 22 | TC must be equipped with a GPRS communication kit |
| 23 | The TC must be equipped with a device for synchronizing time using GPS signals |
| 24 | Protection against electric shock. |
| 25 | TC must be capable of allowing a traffic controller to directly manage the display of traffic signal countdown timers at a traffic signal location using the RS-485 interface and the communication protocol specified in Note** |
| 26 | TC must monitor whether red traffic light signals have burned out and automatically switch the traffic light system to flashing yellow (FY) mode if all red signals for a separate direction (primary and backup) have burned out. |
| 27 | TC must prevent the simultaneous activation of traffic signal combinations that are not provided for in the programmed traffic control scheme a safeguard that, in the event of a conflict, switches the system to traffic signal shutdown mode (OS). |
| 28 | Any malfunctions of the TC must be logged, with the results stored in non-volatile memory. Access to the information can be obtained via a laptop, by transferring the data to a flash drive, or via Bluetooth to a mobile phone. |
| 29 | The startup time of the TC drive must not exceed 20 seconds after power is applied across the entire operating temperature range |
| 30 | The controller housing must be made of vandal-resistant stainless steel with a paint finish that will ensure the controller can operate in urban networks for at least 10 years. |
| 31 | The average time between failures is at least 12,000 hours. |
| 32 | The controller has a service life of at least 10 years. |
| 33 | The warranty period is at least 3 years from the date of commissioning. |

The traffic controllers are planned to be connected to the automated traffic control system currently in operation in Kharkiv; therefore, the proposed controller must support the existing data exchange protocol.

This requirement stems from the fact that the automated traffic control system currently in use in Kharkiv is based on an existing open protocol. Purchasing controllers that do not support the existing data exchange protocol could necessitate reprogramming the entire system, which in turn would result in unnecessary and unjustified expenses for the company.

The participant must use an open data exchange protocol between the traffic controllers and the Central Control Center via GPRS that supports the current data exchange protocol.

Current open protocol for exchanging information between the TC and the Central Control Panel via GPRS

Key Terms and Abbreviations

CONTROL TYPE:

CC - Coordinated Control

DC - Dispatch Control

LC - Local Control

MC - Manual Control

OPERATING MODE:

TLO - traffic light off

FY - flashing yellow

AR - all red

DP – daily program

BDP – backup daily program

SP – special phase

FDP – fixed daily program; differs from DP (coordinated) in that the order and duration of phases are taken from a local program with the same number.

Protocol Description

Packet Format

The following are used for communication via the current protocol: radio channel, Ethernet, and GPRS.

The baud rate is 1200 baud; the channel capacity is 8 bits, including a parity bit and

1 stop bit;

General format:

| marker | Address 1 | Address 2 | command code | length | data | CS |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Field 1: 1 byte long, code - 0x3A, start of text.

Field 2: 1 byte long, network address of the recipient (FF - broadcast to all).

Field 3: 1 byte long, network address of the source subscriber.

Field 4: 1 byte long, command code.

D7 in the command code byte: 0 = request; 1 = response.

Field 5: 1 byte long, length code.

Field 6: 0 to 255 bytes long, data.

Field 7: 1 byte long, checksum (XOR of fields 2–6).

##### Service Commands

##### List of Service Commands

| Command Code | Command description |
| --- | --- |
| 0x03 | RESTART |
| 0x51 | CONNECTIVITY TEST |
| 0x52 | SET THE TIME |
| 0x53 | READ THE TIME |

##### Descriptions of Service Commands

| Command | 0x03 - RESTART |
| --- | --- |
| Answer | 1 byte: most significant quartet: 0 - command accepted but not executed 1 - command executed 2 - command not supported by this controller type 3 - rejected (a higher-priority command is being executed) least significant quartet: reserved |

| Command | 0x51 - CONNECTIVITY TEST |
| --- | --- |
| Length | 10 bytes |
| Data | arbitrary |
| Answer | resubmits the request |

| Command | 0x52 - SET THE TIME |
| --- | --- |
| Length | 7 bytes |
| Data | seconds, minutes, hours, day of the week, date, month, year. (in binary-decimal notation) |
| Answer | length - 0 bytes |

| Command | 0x53 - READ THE TIME |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 7 bytes Data: seconds, minutes, hours, day of the week, date, month, year. (in binary-decimal format) |

##### Status Query Commands

##### List of Status Query Commands

| Command Code | Command description |
| --- | --- |
| 0x60 | GENERAL CONDITION SURVEY |
| 0x70 | GENERAL CONDITION SURVEY (WITH TIME STAMP) |
| 0x42 | STATUS OF OUTPUTS (CHANNELS) |
| 0x66 | CONFLICTS IN DIRECTIONS |
| 0x67 | CHANNEL BURN-OUT |
| 0x68 | NUMBER OF BURNED-OUT LAMPS |

##### Descriptions of Service Commands

| Command | 0x60 - ОПИТУВАННЯ ЗАГАЛЬНОГО СТАНУ |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 5 bytes Data: 1 byte: D7...D6 - control type 0b00 - CC 0b01 - DC 0b10 - LC 0b11 - MC D5.D3 - operating mode 0b000 - TLO 0b001 - FY 0b010 - AR 0b011 - DP 0b100 - BDP 0b101 - SP 0b110, 0b111 - reserve 0b110 - FDP D2 - cycle correction 1 - The start of the controller cycle does not coincide with the specified time stamp. The controller will operate using minimum phase durations until the discrepancy is resolved. 0 - No correction is performed. D1 - Phase clock 0 - Main clock 1 - Intermediate clock D0 - Most significant bit of the DP number 2nd byte: D7…D4 - Current DP number 0x00.0x1F - DP number 1...32 D3.D0 – current phase number 0x0.0xF – phase number 1...16 3rd byte: duration of the current main/intermediate cycle in seconds 4th byte: D7 – door open D6 – conflict D5 – red wire break D4 – burnt-out lamps present D3 – no ~220V mains power D2 – internal controller malfunction D1 - PCP2 stuck (1 - button stuck, 0 - released, if PCP phase is present) D0 - PCP1 stuck (1 - button stuck, 0 - released, if PCP phase is present) 5 bytes: D0 – start of program cycle flag (Transition from AR to phase; if “1,” a program cycle is in progress, and there was no previous phase. This state may occur when transitioning to a cyclic program after a TLO or YF. D4 – controller communication line (CL) error Other bits – reserved |

| Command | 0x70 - GENERAL CONDITION SURVEY (WITH TIME STAMP) |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 8 bytes Data: 1…3 bytes: Snapshot of the time at the moment of formation 1st byte: seconds in DCB 2nd byte: minutes in DCB 3rd byte: hours in DCB 4…9 bytes – status information. The format is the same as the response to command 0x60. |

| Command | 0x42 - STATUS OF OUTPUTS (CHANNELS) |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 6(8) bytes (data length depends on the TC model) Data: 1…6(8) bytes—bit field representing the states of all channels 1 byte: channels 1–8 D7—8th channel … D0 – 1st channel 2 bytes: channels 9…16 D7 – 16th channel … D0 – 9th channel |

| Command | 0x66 - CONFLICTS IN DIRECTIONS |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 2 bytes Data: 1–2 bytes: conflict output numbers (2 numbers); if no conflict is detected, the 1st and 2nd bytes are 0 |

| Command | 0x67 - CHANNEL BURN-OUT |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 8 bytes Data: 1–8 bytes: channel burnout. “Bit field” (64 channels) 0 - channel is intact 1 - channel has burned out |

| Command | 0x68 - NUMBER OF BURNED-OUT LAMPS |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 0 to 32 bytes Data: By tetrad: 0 - All lamps in the channel are intact 1..14 - Number of burned-out lamps 0xF - All lamps are burned out The channel number is determined by the tetrad number. |

##### Control Commands

##### List of Control Commands

| Command Code | Command description |
| --- | --- |
| 0x61 | SWITCH TO TIME MODE |
| 0x62 | SWITCH TO IMMEDIATE MODE |
| 0x63 | SET TO PHASE |
| 0x64 | SWITCH TO LC |
| 0x6D | PERIPHERAL CONTROL |

Descriptions of Control Commands

| Command | 0x61 - SWITCH TO TIME MODE |
| --- | --- |
| Length | 4…28 bytes |
| Data | 1–3 bytes – time to enter mode 1st byte: seconds in DCB 2nd byte: minutes in DCB 3rd byte: hours in DCB 0xFFFFFF – execute upon expiration of Tmin 4th byte: D7 – control type 0 – Manual Control 1 – Remote Control D6...D5 - operating mode 0b00 - TLO 0b01 - FY 0b10 - AR 0b11 – DP (FDP) D4...D0 - DP (FDP) number 0x00.0x1F - DP 1...32 If the bytes listed below (5th and subsequent) containing the phase numbers by tetrads and the phase duration values on a byte-by-byte basis are missing, the DP mode is considered to be FDP mode—this is similar to DP, but with the phase sequence and duration taken from the list of fixed daily programs of the controller with the corresponding number, and only if the specified fixed program is cyclic. Otherwise, the command is not executed. Otherwise, the bytes listed below specify the order of phase rotation and their durations. 5...(4+N) bytes: phase numbers by tetrads, in the order of alternation in the cycle 0x0...0xF - phases 1…16 (5+N)…(4+K) bytes: duration of the phases in bytes N = number of phases / 2 K = N + number of phases |
| Answer | Length: 1–9 bytes Data: 1 byte: most significant quadruplet: 0 – command accepted (the number of phases is in the least significant quadruplet) 1 – command executed (the number of phases is in the least significant quadruplet) 2 – command not supported by this controller type 3 – rejected (a higher-priority command is being executed) 4 - rejected (number of phases exceeds the controller’s specification) 5 - rejected (Tmin > Tphase, in phase X) least significant quartet: (not present in FDP mode) Interpreted depending on the value in the most significant tetrad: most significant; 0..1; 2..4; 5. lower; Number of phases 0x0...0xF - 1...16 phases; 0b0000; Phase X number 0x0.0xF - Phase 1...16. 2...(N+1) bytes: (not present in FDP mode) Phase numbers by tetrads, in the order they occur in the cycle (present only if the most significant byte of the tetrad is 0...1) 0x0...0xF - Phases 1...16 N = number of phases / 2 |

| Command | 0x62 - SWITCH TO IMMEDIATE MODE |
| --- | --- |
| Length | 1...25 bytes |
| Data | The data format is similar to that of command 0x61, except that it lacks the 3 bytes specifying the power-on time. |

| Command | 0x63 - SET TO PHASE |
| --- | --- |
| Length | 4 bytes |
| Data | 1...3 bytes - time to enter mode 1 byte: seconds in DCB 2 bytes: minutes in DCB 3 bytes: hours in the DCB 0xFFFFFF - proceed with the transition immediately 4 bytes: D7...D4 - reserved D3...D0 - phase number (0x0...0xF - phases 1...16) |
| Answer | Length: 1 byte Data: 1 byte: most significant quartet: 0 - command accepted 1 - command executed 2 - command not supported by this controller type 3 - rejected (a higher-priority command is being executed) least significant quartet: reserved |

| Command | 0x64 - SWITCH TO LC |
| --- | --- |
| Length | 0 bytes |
| Answer | Length: 1 byte Data: 1 byte: most significant quartet: 0 - command accepted 1 - command executed 2 - command not supported by this controller type 3 - rejected (a higher-priority command is being executed) least significant quartet: reserved |

| Command | 0x6D - PERIPHERAL CONTROL |
| --- | --- |
| Length | alternate (alternative command) Data field format for the alternative command 1; 2; 2; 2; 2; 2. 1. subcommand (remains unchanged in the response) 2. data (parameters, if a request) By default (unless explicitly stated in the description), for multibyte response values, the first byte received (i.e., the byte with the lowest index in the response buffer) is considered to contain the least significant byte of the value, and so on. Within a byte, the least significant bit corresponds to the least significant address. All subcommand codes are specified in hexadecimal form. |
|  | Subcommand 01 - Management of Recommended Speed Signs |
| Request | 2 bytes 1st byte - speed for the first sign (in HEX) 2nd byte - speed for the second sign (in HEX) Example request: 6D 03 01 32 3C - set the display to 50 km/h for the first sign and 60 km/h for the second |
| Answer | Length: 1 byte 1 byte: most significant quartet: 0 - command accepted 1 - command executed 2 - command not supported by this controller type least significant quartet: reserved |

Alternative Commands 34 (hex)

Alternative Command Data Field Format

| 1 | 2 | 2 | 2 | 2 | 2 |
| --- | --- | --- | --- | --- | --- |

##### 1. subcommand (remains unchanged in the response)

##### 2. data (parameters, if a request)

##### By default (unless explicitly stated in the description), for multibyte response values, the first byte received (i.e., the byte with the lowest index in the response buffer) is considered to contain the least significant byte of the value, and so on. Within a byte, the least significant bit corresponds to the least significant address.

##### All subcommand codes are specified in hexadecimal form.

##### List of subcommands for the alternative command

| Command Code | Command description |
| --- | --- |
| 0-29 | Reserve |
| 30 | Network address and protocol |
| 31 | Controller status and pin mirror |
| 32-49 | Reserve |
| 50 | Advanced status poll |
| 52-59 | Reserve |
| 5A | Reading voltage sensors |
| 5C | Reading port bits |
| 5D - FF | Reserve |

##### Descriptions of the subcommands of an alternative command

| Command | 30 – Network address and protocol |
| --- | --- |
| Parameters | - |
|  | 1st byte - network protocol 2nd byte - network address (least significant byte) 3rd byte - network address (most significant byte) |
| Note | A network address can be a single byte |

| Command | 31 – Controller status and pin mirror |
| --- | --- |
| Parameters | - |
| Answer | 1st and 5th bytes—identical to the five-byte response to the “0x60—GENERAL STATUS QUERY” command 6th byte—the nominal number of controller channels 7th–14th bytes—an 8-byte, bit-by-bit channel status mask. A set bit indicates that the channel is enabled. |

| Command | 50 – Advanced status poll |
| --- | --- |
| Parameters | - |
| Answer | 1st byte D7…D6 Control type 00 - LC 01 - DC 10 - CC 11 - MC D5 - No alignment data (1 - no data) D4 - Power Switch (1 - power switch off) D3 - Intermediate cycle flag (1 - intermediate cycle) D2 – Fault indicator leading to a change in operating mode (These include faults such as green wire conflict, red wire break, short circuit, overvoltage or undervoltage, and others. 1 – fault present, 0 – no fault) D1...D0 Operating mode 00 – Control mode (not TLO, not FY, not AR) 01 – TLO (channels off, 1 – channels off) 10 - FY 11 - AR 2nd byte: D7 - SET1 toggle switch D6 - SET2 toggle switch D5 - SET3 toggle switch D4 - SET4 toggle switch D3 - TLO toggle switch D2 - FY toggle switch D1 - door D0 - indicator of overvoltage or undervoltage (1 – overvoltage or undervoltage; 0 - voltage within normal range) 3rd byte: D7 - TPV1 logical D6 - TPV1 physical D5 - TPV2 logical D4 - TPV2 physical D3.D0 - source of current control mode 0000 - No “external” control (controller in local control mode) 0001 - Local control center 0010 - Radio control system 0011 - START control system 0100 - Control panel 0101 - Toggle switches 0110 - External toggle switches 0111 – Police console (special-purpose console) 4th byte: D7 - reserved D6 - reserved D5 - reserved D4 - reserved D3 - reserved D2 - reserved D1 - reserved D0 - Cycle adjustment flag (1 - cycle adjustment is performed, 0 - cycle adjustment is not performed) 5th byte: program number (0 - no program, 1–24 - program number) 6th byte: phase number (0 – no phase, 1–16 – phase number) 7th byte: current program stage time (upward count for Tosn, downward count for industrial cycle) 8th and 9th bytes: mains voltage 10th and 11th bytes: battery voltage Bytes 12, 13, 14, 15, 16, 17, 18: seconds, minutes, hours, day of the week, date, month, year in binary-decimal format Byte 19: number of open red lines (0 – no open lines) Bytes 20, 21: numbers of conflicting green channels (if both are zero, there are no conflicts; if there is a conflict in one channel, the least significant byte contains the number of the conflicting channel and the most significant byte is 0; if there is a conflict in two channels, both bytes contain the numbers of the conflicting channels; 1 – first channel) Byte 22: number of the channel in which a short circuit is detected (0 – no short circuit, 1 – first channel) Bytes 23–30: bit mask of the control status in each channel (D0 – lower channel, 0 – off, 1 – on) Bytes 31–38: bit mask for the current value in each channel (D0 – lower-order channel, 0 – off, 1 – on) Bytes 39–46: bit mask for the voltage value in each channel (D0 – lower-order channel, 0 – off, 1 – on) |

| Command | 5A - Reading voltage sensors |
| --- | --- |
| Parameters | 1st byte - request type 0 - mains and battery voltage 1 - mains and battery voltage and channels 1–32 2 - mains and battery voltage and channels 33–64 3 - voltage of channels 1–32 4—voltage of channels 33–64 |
| Answer | Single-board: an array of two-byte values as specified in the request 1st pair - mains voltage 2nd pair - battery voltage (channel voltages are listed in ascending order of channel numbers) SPI - KDK: an array of two-byte values as per the request 1st pair - mains voltage 2nd pair - battery voltage TWI - KDK: walk-by (no response) |

| Command | 5C - Reading port bits |
| --- | --- |
| Parameters | - |
| Answer | 1st byte: 1st bit - SET1 switch 2nd bit - SET2 switch 3rd bit - SET3 switch 4th bit - SET4 switch 5th bit – TLO toggle switch 6th bit – START line status. STARTIN 7th bit – START line status. DATAIN 8th bit – FY |
|  | 2nd byte: 1st bit – RXDI signal status 2nd bit – DET1 signal status 3rd bit – PCP1 4th bit – PCP2 5th bit – door |

** Note: Description of the communication protocol between traffic controllers and countdown timers for traffic lights.

Controlled countdown displays are designed to show the remaining duration of a phase when phase durations vary within a cycle. Phase durations may change during adaptive control, particularly when controlled by a pedestrian call button (PCB).

Control is implemented by transmitting the number of seconds displayed on the countdown display via the RS-485 interface. No response is sent from the countdown display.

The protocol specifies that each countdown display has its own address. The protocol syntax allows for addressing up to 32 countdown displays.

Byte format

Transmission speed: 115,200 baud

Transmission method: asynchronous

Number of bits: 8

Stop bits: 2

Parity check: odd

Frame Format

| № | Meaning | Description |
| --- | --- | --- |
| 0 | FF | Fixed values |
| 1 | 3A |  |
| 2 | 00 |  |
| 3 | 3E |  |
| 4 | 83 |  |
| 5 | 22 |  |
| 6 | 34 |  |
| 7 | Digit 1 | Count value for the first address on the display |
| … | … |  |
| ... | ... |  |
| 38 | Digit 32 | Count value for the 32 address on the display |
| 39 | Brightness | The Brightness Levels of Traffic Light Signals |
| 40 | CRC | Checksum |
| 41 | FF |  |

Data is transmitted in HEX (hexadecimal format).

Digit (Countdown value) ranges from 1 to 99 seconds (1–63 in HEX). If a countdown is not required for this traffic light, the value is 0.

Brightness (Brightness value) ranges from 0 to 255 (0–FF in HEX). May not be used.

CRC (Checksum). Calculated by bitwise addition modulo 2 (XOR) of the number “AB” and the values of fields 7–39.